# MSCQR Printing Architecture Standard

## Purpose

This document defines the production printing standard for MSCQR. The goal is simple:

- no manual "run the agent" step for end users

- no browser-only local printer hacks

- no insecure exposure of printer ports to the public internet

- one secure print workflow across workstation, LAN, and private-site printers

## Why pure browser local-printer printing is intentionally not supported

Browsers do not expose a production-safe, silent, printer-aware local printing API for this use case.

- window.print() opens the browser print dialog and cannot enforce secure printer selection, quiet background dispatch, or device capability checks.

- Browser hardware APIs such as WebUSB, WebSerial, and WebHID are not the right production foundation for mixed printer fleets. They require per-browser support, explicit user permissions, and device-specific handling.

- A web app cannot reliably inspect the operating system printer list, validate spooler readiness, or manage secure retries by itself.

For that reason, MSCQR does not treat pure browser local-printer printing as a supported production path.

## Approved production patterns

### 1. LOCAL_AGENT

Use this for:

- USB printers

- driver-dependent printers

- workstation-managed Wi-Fi or office printers

- printer languages that still depend on the OS driver path

Operational standard:

- install the signed MSCQR Workstation Connector once

- auto-start at login

- keep it running in the background

- let the browser communicate only with the trusted local listener

### 2. NETWORK_DIRECT

Use this for:

- raw LAN label printers

- stable IP/port printer targets

- approved command languages only

Current supported command languages:

- ZPL

- TSPL

- EPL

- CPCL

This mode is intentionally restricted to registered printer profiles. Freeform host entry during print is not allowed.

### 3. NETWORK_IPP

Use this for:

- AirPrint printers

- IPP Everywhere printers

- office printers that accept PDF over IPP/IPPS

Operational standard:

- prefer ipps:// when the printer supports TLS

- validate the printer URI before dispatch

- submit standards-based PDF jobs over IPP/IPPS

- do not downgrade AirPrint printers to raw port 9100

## Private-LAN site gateway standard

When the backend cannot directly reach a manufacturer-site printer, MSCQR uses a site gateway pattern:

- the installed workstation connector keeps an outbound authenticated connection to MSCQR

- MSCQR never requires inbound firewall holes to reach the site

- the gateway claims approved print jobs and dispatches them locally

- gateway credentials are provisioned once through the printer profile bootstrap flow

This is the approved pattern for private manufacturer networks.

## Dispatch routing standard

MSCQR routes printers by capability:

1. LOCAL_AGENT for workstation-managed printers

2. NETWORK_DIRECT for raw ZPL/TSPL/EPL/CPCL label printers

3. NETWORK_IPP for AirPrint / IPP Everywhere printers

## Security requirements

- approved labels remain server-controlled

- print locks and render-token workflows remain intact

- printer targets must be registered, validated, and auditable

- raw printer ports are never exposed publicly

- gateway credentials are scoped to registered printers

## Operational guidance

- Fleet rollout should use signed installers or MDM, not terminal instructions for operators.

- For workstation printing, the OS must already see the printer before MSCQR can use it.

- For NETWORK_IPP, validate PDF support and URI reachability before the first live job.

- For private-LAN sites, prefer NETWORK_IPP with site gateway over ad hoc workstation browser flows.